Cie 3 notes

- Semiconductors allow a controlled flow of electrons, forming the core of electronic devices.

- Before transistors (invented in 1948), vacuum tubes (valves) like vacuum diodes were used.

- Adv include:

  - Bulky, high power consumption, operated at ~100 V.

  - Low reliability and short lifespan.

- In the 1930s, it was discovered that solid-state semiconductors could control charge carriers.

- Semiconductors work without needing vacuum or heating and respond to light, heat, or small voltage changes.

- Advantages:

  - Compact size, low power consumption, low voltage operation, high reliability, and long life.

- Applications

  - Photovoltaics, Photodetectors, LEDs, Batteries, SC

- Important for sustainable energy solutions (e.g., generation and storage).

Classification

- Elemental semiconductors (Si, Ge).

- Compound semiconductors

  - Inorganic: CdS, GaAs, CdSe

  - Organic: fullurene, anthracene

  - Organic polymers: polypyrrole, polyaniline

Classification of Inorganic Semiconductors

- Intrinsic Semiconductors

- Extrinsic Semiconductors

Intrinsic Semiconductor

- Definition: Pure semiconductors with no impurities in the crystal.

- Common examples: Silicon (Si) and Germanium (Ge), which have a diamond-like crystal structure.

- Each Si/Ge atom has 4 valence electrons and shares them with 4 nearest neighbours, forming covalent bonds.

Properties:

- At T = 0 K: behaves like an insulator.

- At T > 0 K: thermal energy excites some electrons from valence band (Ev) to conduction band (Ec).

- This creates electron hole pairs.

- Number of free electrons (ne) = Number of holes (nh).

- Denoted as: ne = nh = ni (where ni is intrinsic carrier concentration).

Extrinsic Semiconductors

- Definition: Doping of a pure (intrinsic) semiconductor (having low conductivity at room temp), with small amounts of impurities (dopants) to increase its electrical conductivity.

- The process of adding these dopants is called doping.

- Depending on the type of dopant:

  - If doped with pentavalent atoms (valency 5), it becomes an n-type semiconductor (extra electrons are majority carriers). e.g., As, Sb, P

  - If doped with trivalent atoms (valency 3), it becomes a p-type semiconductor (holes are majority carriers). e.g., In, B, Al

n-type Semiconductor

- How it works: Doped with pentavalent atoms (e.g., As).

- 4 electrons bond with silicon; 5th electron remains free.

- This extra electron becomes a charge carrier.

- Energy required to free this electron is very low (e.g., 0.01 eV for Ge, 0.05 eV for Si).

- These atoms are called donor impurities.

Characteristics:

- Electrons are majority carriers.

- ne (electron concentration) >> nh (hole concentration)

- Conduction is due to free electrons from dopants and a few from intrinsic excitation.

p-type Semiconductor

- How it works: Doped with trivalent atoms (e.g., B).

- Can bond with only 3 neighboring Si atoms.

- Leaves a hole (vacancy) where the 4th bond should be.

- Electrons from neighboring atoms may jump to fill the hole, creating mobile holes.

Characteristics:

- These dopants are called acceptor impurities.

- Holes are majority carriers, electrons are minority carriers.

Production of Electronic Grade Silicon (EGS)

- Purpose: To produce high purity, single crystal silicon from bulk polycrystalline material for semiconductor devices.

- Method: solidifying silicon from its molten state. (Melt growth)

Crystal Growth Techniques

- Czochralski Method

- Float Zone Method

Czochralski Method

- Overview: A technique to grow single crystal silicon ingots (boules).

- These boules are sliced into thin circular wafers and then diced into little Si chips from which large scale integrated circuits are made.

Requirements:

- Furnace:

  - Includes fused silica crucible (SiO2), graphite susceptor, heating mechanism (resistance or RF), power supply and rotation system.

- Crystal Pulling Mechanism:

  - Includes seed holder, counterclockwise rotation

- Ambient Control:

  - Oxygen exclusion using Argon gas. Graphite parts can react with O2, forming CO2 which must be avoided.

- Control System:

  - Microprocessor based. Controls temperature, crystal diameter, pull rate, rotation speed

Working Principle:

- Melting: Polycrystalline silicon (the feed material) is melted at 1410°C in a crucible by resistance or radio frequency heater.

- Seeding: A seed crystal is dipped into the saturated molten silicon.

- Crystallization: Seed rotates counterclockwise, and the crucible rotates clockwise.

- Crystal grows as the seed is slowly pulled upward.

- Shape Control: Achieved by adjusting heating power, pull rate and rotation speed

Final Product:

- Crystals up to 300 mm diameter, 300 kg weight

- Most imp technical application is growth of dislocation free silicon which is crucial in high performance electronics.

Float Zone (FZ) Method

Why FZ Over Cz?

- Cz wafers contain oxygen impurities due to contact with the SiO2 crucible, reducing minority carrier lifetime and inturn reducing efficiency of solar cells.

- Oxygen and its complexes may become active in higher temp making wafers sensitive to high temperature processing.

- FZ wafers are preferred in high efficiency or lab grade solar cells where purity is critical.

Working:

- It is a crucible free process i.e. no container touches the silicon.

- Surface tension maintains the molten silicon zone to be kept between the two vertical solid rods.

- A seed crystal is dipped into one end of the melt.

- The molten zone is moved along the feed rod.

- At the other end, crystal growth occurs as the molten silicon solidifies.

Advantages

- Avoids contamination from crucible materials.

- High purity crystals.

- Minimal crystal defects due to lack of container interaction.

Disadvantage of FZ

- Size limitation: Max wafer diameter ≈ 200 mm (compared to up to 300 mm in Cz).

- Surface tension limits the zone width.

- This restricts its use to specialized applications, not mass production.

GaAs

General Overview

- GaAs (Gallium Arsenide) is a III-V compound semiconductor.

- Widely used in optoelectronics (LEDs, laser diodes) and microelectronics (high frequency ICs).

- Has FCC cubic structure, specifically zinc blende.

Electrical Properties

- Poor intrinsic conductivity makes it semi insulating in pure form.

- Doping with p-type or n-type impurities allows conductivity for device applications.

- High mobility and saturated drift velocity making it better than silicon for high freq applications.

- GaAs velocity peaks then drops (due to intervalley scattering), while Si saturates more gradually.

Optical Properties

- It is a direct bandgap semiconductor (1.42 eV)

- Direct bandgap means that the min of the CB is directly over the max of the VB.

- Transitions across the VB n CB require only a change in energy and no change in momentum

- This leads to efficient photon emission making it ideal for LEDs and laser diodes.

- Only GaAs and InP are suitable for efficient light emission.

Semi-Insulating (SI) GaAs

- Used as substrates for Integrated circuits due to high resistivity and electrical isolation.

- Formed by either intentional doping with transition metal impurities during growth from melt or by relying on the formation of native defects during crystal growth.

- SI GaAs is mostly used as substrate for devices made by ion implantation method since it reduces parasitic cap0061citances.

- Key features include high resistivity, high mobility, thermal stability and low crystalline defects.

Applications of GaAs

- High freq app

  - Ideal for GPS, WLAN, and laser diodes.

  - Works well at high speeds and high temperatures.

- Transistors and Computer:

  - Used in transistors and logic circuits.

  - Faster electron transport than in silicon giving better performance.

- Defence and Aerospace

  - Used in radars, sensors, and secure comms.

  - Radiation resistance making it suitable for space applications.

Silicon Germanium (Si₁₋ₓGeₓ) Summary

Introduction

- Si(1-x)Ge(x) alloys have been researched since the 1950s, with practical applications rising in the past 30 years.

- Used initially in bipolar technologies, now common in MOS (metal oxide semiconductor) technologies.

- Silicon and germanium are completely miscible over the full range of x (0 to 1).

- The structure is diamond like cubic (same as silicon).

Thermoelectric Properties

- Thermoelectric effect: In presence of temperature gradient generates electric voltage and visa versa.

- Depends on the thermoelectric material figure of merit (zT)

- S: Seebeck coefficient, σ: Electrical conductivity, κ: Thermal conductivity, T: Temperature

- For high efficiency, high S and σ, low κ are preferred.

Performance Highlights:

- Operates up to 1050°C without degradation.

- Doping (e.g., phosphorus (n-type), boron (p-type)) adjusts carrier concentration.

- Used in Voyager missions for over 40 years in space power systems (RTGs).

- Strategies like using 0D (clusters), 1D (nanotubes), 2D (layers) improve thermoelectric performance by reducing dimensionality.

Band Gap

- SiGe is an indirect band gap semiconductor.

- Has a narrower band gap than pure silicon.

- The band gap is tunable by changing germanium content (x).

Doping & Fabrication

- SiGe can be doped p- or n-type when amorphous.

- Doping is difficult in crystalline SiGe due to high crystallization temperatures.

- Challenges in growing high quality doped films for mass production.

Applications

- Space Industry: RTG generators in deep space missions (e.g., Voyager).

- Automotive: Ideal for rectifiers in high temp environments.

- Electronics & Telecommunication:

  - Used in Heterojunction Bipolar Transistors (HBTs).

  - Enables bandgap engineering within silicon systems.

- Solar Industry:

  - 4× faster carrier mobility in comparison to silicon.

  - Reduces energy loss, allows smaller, faster solar cells.

- Optical Interconnects:

  - High speed detection, low cost, suitable for IC interconnects in optical devices.

Indium Phosphide (InP)

- Material Class: Group IIIA phosphide nanocrystalline semiconductors.

- Key features include large direct band gap, strong covalent bonding and less ionic character.

- Used in high speed digital, microwave, and optoelectronic devices.

Optoelectronic Properties

- Photoluminescence: It is an optical phenomenon that semiconductors give light emissions by absorbing photons with energy higher than its energy band gap.

- Quantum Dots (QDs):

  - Size tunable optical band gap covering visible spectrum.

  - Non-toxic alternative to Cd- and Pb-based QDs.

- Band Gap: 1.35 eV, suitable for both visible and near infrared light.

- Heterojunction Compatibility: InP works well with ZnS, ZnSe, and Graphene for various device applications like displays, solar cells, and photodetectors.

Applications of InP

- InP Nanowire-Based Solar Cells:

  - Uses core shell pn junction InP NWs.

  - Achieved solar efficiency of 13.8%.

- InP Polymer Hybrid Photodiodes:

  - Enhances carrier collection.

  - Cost effective ITO substrates used.

  - Suitable for optoelectronic sensing.

- InP Nanowires as LEDs:

  - pn junctions emit polarized light under forward bias.

- Field-Effect Transistors (FETs):

  - ZnS sensitized InP NWs enable good p-type transistor behavior.

- Bioimaging:

  - Non-toxic alternative to CdSe QDs.

  - Folic acid–capped InP QDs target tumor cells (e.g., human oral carcinoma).

Organic Semiconductors: Overview

- Definition: Materials that combine electrical adv of semiconducting materials with the chemical/mechanical benefits of organic compounds (like plastics).

- Advantages:

  - Absorb and emit light.

  - Conduct electricity.

  - Mechanically flexible, lightweight, and soluble.

  - Easily modified chemically for tailored properties.

- Composition:

  - Primarily made of carbon and hydrogen atoms.

  - May include heteroatoms like sulfur, oxygen, nitrogen.

  - Exhibit properties typical of inorganic semiconductors.

- Conductivity: Originates from:

  - Charge injection at electrodes.

  - Doping (intentional/unintentional).

  - Dissociation of photogenerated electron hole pairs.

Origin of Semiconducting Property in Organic Molecules

- Molecular Structure:

  - Composed of conjugated systems (alternating single and double bonds).

  - π orbital delocalization allows electron mobility within molecules.

- Intermolecular Bonding:

  - Molecules are held together via weak van der Waals interactions.

    - Leads to localized electrons (except in π orbitals) and narrow electronic bandwidths.

- π - π Overlap:

  - Key for electron delocalization.

  - Greater π - π overlap → better charge transport.

  - HOMO-LUMO energy gap analogous to band gap in inorganic semiconductors.

Types of Organic Semiconductors

- Amorphous Molecular Films:

  - Formed by evaporation or spin coating.

  - Used in LEDs, xerography (MDP films).

- Molecular Crystals:

  - Comprise a regular lattice.

  - Molecules like naphthalene, anthracene, etc.

  - High charge mobility makes it suitable for transistors.

- Polymer Films:

  - Long chains of repeating molecular units.

  - Processed from solution (spin-coating, ink jet, reel to reel coating).

  - Blends enhance stability and reduce crystallization susceptibility.

Pentacene

- Structure: A linear acene molecule made of five fused benzene rings.

- Application: Ideal for highly ordered organic thin-film transistors (OTFTs).

- Crystal Structure: Forms molecular crystals with a herringbone packing.

- Electron Transport: Delocalized π orbitals facilitate efficient charge movement via π–π stacking.

- Lattice: Adopts a triclinic lattice structure in bulk.

Synthesis of Pentacene

Step 1:

- Reactant: Pentacene-6,13-dione.

- Solid NaBH4 was added to the round bottom flask containing suspension of THF at 0 °C.

- Then the reaction mixture was purged with N2 and water was added.

- The mixture was heated to 50–60 °C until it becomes homogeneous.

- After 2 hrs o f heating, THF was evaporated at reduced pressure, water was added and the reaction mixture was filtered.

- The solid were washed with water. After drying, a white solid was recovered.

- Product: 6,13-dihydroxy-6,13-dihydropentacene (85% yield).

Step 2:

- Reactant: Dihydropentacene.

- The obtained product was stirred with SnCl2 (1:2 mole ratio) in acetone.

- The resulting crude pentacene was isolated by centrifuge and washed with both methanol and THF.

- Purification of sublimation resulted in pure pentacene (yield ~60%).

Fabrication of Pentacene Thin Films

- It is sensitive to air (moisture, oxygen) and low solubility makes it unstable with standard fabrication methods.

- Thus, pentacene should be deposited using evaporation techniques to fabricate thin films for electronic devices.

- It is an energy intensive process.

- To overcome this, substituents are introduced to the aromatic core, which increases the stability, solubility, and packing for better electrical properties. of the pentacene.

Thermal Evaporation Technique

- Organic molecular beam deposition (OMBD) one of the thermal deposition technique is used to produce high quality thin films of pentacene.

- Uses ultra high vacuum to evaporate material.

- Molecules are sublimated and deposited on a substrate which is placed parallel to the sublimed molecules.

- Advantages include high control over film thickness and order.

Spin Coating:

- Simple and widely used due to its capacity for mass production and cheaper costs.

- A solution is dropped on a spinning substrate which starts spinning with a certain acceleration rate and remains at const. speed for a period of time.

- Thin film is formed as solvent evaporates.

App of pentacene

- Solar Cells

  - Used as an active layer in donor acceptor solar cells.

  - Pentacene was stacked with PTCDI  and this layer is deposited onto PEDOT-PSS layer via thermal evaporation  (3 × 10⁻⁷ Torr, 1 Å/s) to obtain 50 nm of pentacene.

- Photodetector

  - Used in flexible photodetectors.

- Organic Thin Film Transistor (OTFT)

  - Common in organic field effect transistors (OFETs) due to high charge mobility.

- OLED (Organic Light Emitting Diode)

  - Used as a component to improve current efficiency in OLEDs.

Fullerene

- Definition: Fullerenes are allotropes of carbon, forming hollow molecular structures either spherical (e.g., C₆₀, "buckyballs") or cylindrical (carbon nanotubes)

- Named acc to the number of carbon atoms in the structure.

C₆₀ Molecule (Buckminsterfullerene):

- 60 carbon atoms, diameter ≈ 7.09 Å

- Shape: Truncated icosahedron (soccer ball like)

- Each vertex has one carbon atom, and each carbon has 1 double bond and 2 single bonds

- Contains 12 pentagonal rings

- Can be viewed as a rolled up graphene sheet

Properties:

- Act as electron acceptors due to low energy and triply degenerate LUMO (easily accepts electrons)

- Exhibit low reorganization energy, leading to efficient charge separation, reduced recombination, and long lived charge separated states.

- High spherical symmetry supports good electron delocalization and stability.

- Fullerenes and derivatives act as n-type materials due to excellent electron affinity.

Synthesis of PCBM

- Two-Step Process:

- [2+1] Cycloaddition reaction of C₆₀ with diazoalkane to form fulleroid intermediate

- Isomerization converts the fulleroid into PCBM by thermal or photoisomerization

App of fulerenes

- Solar Cells

  - Efficient electron acceptors making them promising candidates for construction of solar panels.

- Organic Thin Film Transistors (OTFT)

  - Used as active semiconducting material.

  - Materials: Fullerene and pentacene often used due to high electron/hole mobility.

- Batteries and Supercapacitors

  - Electrochemical intercalation of lithium leads to C₆₀Li₁₂ formation.

  - Fullerene (C₆₀) and its derivatives used for anode materials in lithium ion batteries.

- Sensors

  - Due to high conductivity, charge transport, and separation abilities, fullerenes are ideal for strain sensors, gas sensors, photoelectrochemical sensors, electrochemical and optical sensors

- Biomedical Applications

  - Excellent electrochemical sensitivity of C₆₀.

  - Used in detection of biomolecules: dopamine, glucose, uric acid, etc.

- Superconductors

  - When alkali metals (e.g., potassium) are embedded in C₆₀, the compound becomes a superconductor. eg K₃C₆₀

  - It possesses high transition temperature, 3D superconductivity, good current density, ductility, and stability

Conductive polymers

- Traditionally, polymers were insulators, but a special class called Intrinsically Conducting Polymers (ICPs) or Organic Conductive Polymers can act as semiconductors or conductors.

Key Characteristics:

- They contain alternating single and double bonds (conjugated systems) or aromatic rings (e.g., phenylene, thiophene).

- First significant conducting polymer: Polyacetylene (polyethyne).

- Examples of Conductive Polymers include Polyacetylene, Polyaniline etc

Properties:

- High Melting/Softening Points due to restricted mobility from aromatic ring structures and absence of free rotation.

- Excellent Stability: chemically, thermally, and oxidatively stable due to low hydrogen content and aromatic nature.

- Processability: can be fabricated into highly ordered, crystalline thin films that conduct electricity when doped.

Mechanism of Conductivity:

- Conductive polymers rely on delocalized π electrons in conjugated systems.

- These create a band structure like semiconductors.

- Doping (p-type or n-type) introduces charge carriers by removing electron from VB by oxidation (p doping) or adding electrons to CB by reduction (n doping).

- Delocalization of electrons across the π system enhances conductivity.

Polyaniline (originally called aniline black).

- It is a mixed oxidation state polymer containing benzoid and quinoid units.

- It has characteristics of switching between conductive and insulating states depending on experimental conditions.

- PANI exists in three oxidation states, defined by the parameter 1−y:

| Form | Oxidation State (1 − y) | Description |
| --- | --- | --- |
| Leucoemeraldine (LE) | 0 | Fully reduced, all benzoid units |
| Emeraldine Base (EB) | 0.5 | Half oxidized, alternating benzoid and quinoid units; most stable and useful form, acts as a semiconductor |
| Pernigraniline (PE) | 1 | Fully oxidized, all quinoid units |

Preparation of (PANI)

- Synthesis Steps:

- Reactants: Aniline and APS (Ammonium Persulfate) in 1.0 M HCl.

- Process:

- Stirred vigorously at ~0 °C.

- Oxidative polymerization occurs forming a dark green PANI powder.

- Post-treatment:

- Filter and wash with 1.0 M HCl and distilled water until clear.

- Final wash with acetone, dry at 60 °C.

Applications of PANI

- Solar Cells:

  - Used in Dye Sensitized Solar Cells (DSSC).

  - Attractive due to low cost, easy synthesis, and conductivity.

- Electrochromic Glasses:

  - Smart glass changes color under electric current.

  - PANI reflects blue and is used to control light transmission (e.g., car windows).

- Electroluminescence Machines:

  - Used in devices that emit light upon electrical stimulation.

- Sensors:

  - Due to redox sensitivity, ideal for detecting gases, biomolecules, etc.

Magnetic Materials for data storing

Principle of Magnetic Recording:

- Data is written on a medium (disk/tape) that contains magnetic layers.

- Uses magnets with north and south poles to encode data.

- Magnetic field sensors detect the magnetic field which allows data reading.

- External magnetic fields change polarity which allows data writing.

Working Mechanism:

- Identical poles (N–N or S–S) create a magnetic field whose voltage detected by GMR sensor which represents “1”.

- Opposite poles (N–S) do not produce a field. Thus no voltage and represents “0”.

Materials Used:

- Elements: Iron (Fe), Nickel (Ni), Cobalt (Co).

- Alloys: Co–Pd, Pt–Fe nanoparticles.

- Thin films: Co–Ni–Pt, Co–Cr–Ta.

- Others: γ-Fe₂O₃, single-atom Ho on MgO, carbon coatings for durability.

Dielectric Materials for Storage Devices

- Role in Memory:

  - Dielectrics are poor conductors but get polarized when voltage is applied.

  - Used in volatile (DRAM) and non-volatile (RRAM) memories.

- RRAM (Resistive Random Access Memory) Working Principle:

  - Structure: Electrode/Dielectric/Electrode.

  - Based on resistance change (switching between high and low resistance).

  - The storage medium uses High Resistance State (HRS) and Low Resistance State (LRS)  to represent 1 and 0 of binary code.

  - Flexible versions use cross array structures to maximize density to improve storage density.

Dielectric Materials Used

- 1. Inorganic Materials:

- Oxides: TiO₂, NiOₓ, ZnOₓ, AlOₓ, WOₓ, HfOₓ, MoOₓ

- Others: HfTiO₄, ZnSnOₓ, Si₃N₄, BN, amorphous Si

- 2. Organic Materials:

- Polymers: Lignin, polyimide (PI), PEGDMA, starch, NC

- Example: pV3D3, polyparaxylylene

- 3. Organic-Inorganic Composites:

- Combinations:

- PVP:GO, ultrathin HfOₓ

- HKUST-1 (MOF: Cu₃(BTC)₂)

Applications of Dielectric Materials:

- Resistive switching memory

- Thin-film transistors

- Film capacitors

- Dielectric actuators

- Capacitive sensors

Photochromic Materials:

- Definition: Substances that reversibly change color when exposed to light, especially UV light.

- They absorb light, undergo a reversible chemical change in structure and color, and return to the original state when the light source is removed.

- Available in glass, plastic, or powder form.

Important Compounds

- Diarylthene (DE), Dithienylethene (DT), Furylfulgide (FF): thermally stable and resistant to photochemical side reactions and used in optical information storage.

- Spirooxazines (SO) and Spiropyrans (SP): Used in optical switches, often embedded in sol-gel glasses.

Working Principle

- It is a reversible photochemical reaction that occurs when material is exposed to light.

- The photochromic material absorbs light energy and undergoes a reversible chemical reaction that changes its molecular structure and color.

- When the light source is removed, the material returns to its original state.

- UV light changes molecular structure.

- UV exposure triggers molecular shift, increasing light absorption and gives the lenses a darkened appearance.

- In absence of UV, molecules revert and become transparent again.

- Photochemical molecules are sensitive to both the presence and amount of UV light.

Photochromic Lenses: Structure & Mechanism

- Transparent indoors and darken when exposed to UV light from sun

- Provide benefits of both glasses and sunglasses in a single frame.

- Plastic Lenses: Have carbon based compounds within them that change structure in presence of UV light.

- Glass Lenses: Contain silver halide crystals (like silver chloride).

- UV light causes silver gains an electron (Ag⁺ → Ag⁰), darkening the lens.

- In absence of UV, a second compound (like copper chloride) reverses the reaction.

- UV light causes a bond to cleave (oxygen-carbon), altering the molecules shape and properties.

Advantages of Photochromic Lenses

- Dual purpose: Act as both prescription glasses and sunglasses.

- Provide full protection from UVA and UVB rays.

Disadvantages

- Unsafe for driving: Lenses may darken behind windshields.

- Some are not polarized, causing glare issues.

Applications of Photochromic Materials

- Photochromic units can be used to create molecules that can switch shape, func or interactions with light.

- Photochromic compounds can be used to store info in optical media by changing a reflectivity with light.

- Photochromic materials can be used to store solar energy by converting light into heat or electricity and releasing it when needed.

- Photochromic dyes can be used to make fabrics that change color with light

Thermochromic Materials:

- Definition: Thermochromic materials change color due to temperature changes.

- Common Applications: Kettles, Coffee cups, Temperature indicators

Types of Thermochromic Materials:

- 1. Liquid Crystals (LCs):

  - Organic compounds that reflect different colors at different temperatures.

  - Examples: Cholesteryl esters, ethers, benzoates, oleyl carbonates.

  - Applications: Thermometers, mood rings, thermal cards.

- 2. Leuco Dyes:

  - Organic molecules that switch between colored and colorless states when they interact with other chemicals at certain temps

  - Examples: Crystal violet lactone, malachite green lactone, phenolphthalein, fluoran.

  - Used in papers, polymers, inks, etc.

- 3. Inorganic Materials:

  - Change optical properties due to change in crystal structure or electronic configuration at certain temps.

  - Include metal oxides like: Titanium dioxide (TiO₂), Zinc oxide (ZnO)

Working Principle of Liquid Crystal Thermochromic Materials:

- These materials reflect light differently depending on temperature and molecular alignment.

- Operate based on selective reflection of certain wavelengths depending on temp and orientation of molecules.

- Exhibit a phase change from crystalline (chiral/twisted nematic phase) at low temp to isotropic (liquid) phase at high temp.

- Only nematic mesophase shows thermochromic properties which restricts the effective temperature range of material.

- Hot: Crystals are closer together, reflects blue light.

- Cold: Crystals spread out, reflects red light.

Applications of Thermochromic Materials:

- Temperature Indicators in Industry (chemical reactions, heat variation)

- Thermometers for rooms, refrigerators etc

- Indicator of level of propane in tanks, which change color depending on temp

- Heat transfer mapping devices which shows heat and convection patterns.

- Artistic and educational materials such as mood rings, thermal cards etc

Electrochromic Materials

- Definition: Electrochromic materials change color or opacity when an electric field is applied.

- Unlike photochromic (light sensitive) or thermochromic (temperature sensitive) materials, electrochromic materials respond to voltage/electric field.

- Applications: Used in smart windows, mirrors, sunglasses, and other devices that manage light and heat transmission.

Types of Electrochromic Materials

- Metal Oxides:

  - Inorganic compounds that undergo color changes by ion/electron exchange.

  - Example: Tungsten oxide (WO₃) changes from transparent to blue when voltage is applied.

- Conducting Polymers:

  - Organic compounds that change color by oxidation state changes.

  - Example: Polyaniline shifts from green to blue.

- Organic Dyes:

  - Undergo redox reactions to change molecular structure and hence color.

  - Example: Viologen changes from colorless to blue or red.

Working Principle

- It is based on reversible change in optical properties caused by redox reactions.

- When the material is placed on the surface of an electrode, an electric field can induce redox reactions and change the color or opacity of the material.

- The color change can be between a transparent state and a color state or between two colored states.

- The color change can be controlled by the mag of the applied voltage.

Device Structure: Electrochromic Device (ECD)

- Mechanism:

  - Electrons and charge balancing ions migrate into EC layer and thus optical absorption occurs.

  - Voltage induces redox reactions, changing color or transparency.

- Shows electron & ion movement between ion storage and EC layers, enclosed by electrodes.

Device Operation Modes:

- 1. Transmittance Mode:

  - Transparent conducting electrodes (TCE) on both sides.

  - Controls light intensity (used in smart windows).

- 2. Reflectance Mode:

  - One TCE replaced by reflective material (e.g., aluminum, silver).

  - Controls light reflection (used in rear-view mirrors).

Applications:

- Auto dimming rearview mirrors

- Smart sunglasses or adaptive goggles

- Sensors, optical shutters/modulators

General Device Structure:

- Illustrates:

  - Movement of electrons (e⁻) and ions

  - Layers: Electrochromic layer, Ion storage, Electrolyte

  - Reversible reaction: MV²⁺ + e⁻ ⇌ MV⁺ (redox process)

Thin Film Deposition Techniques

- Definition: Thin films are layer of material ranging from a few nanometers to several micrometers thick, used in various technological applications.

Classification of Thin Films

- (a) Single crystals

- (b) Epitaxial

- (c) Polycrystalline

- (d) Amorphous

Properties of Thin Films

- High surface to volume ratio

- Geometrical control by substrate

- Compact structure

- Properties similar to single crystals

Uses of Thin Films

- Microelectronic and telecommunication devices

- Decorative and optical coatings

- Sensors and catalysts

Thin Film Deposition Methods

- Physical Vapour Deposition (PVD)

- Chemical Vapour Deposition (CVD)

Chemical Vapour Deposition (CVD)

- Formation of non-volatile solid thin films on substrates by chemical reactions of vapor phase precursors.

- Involves introduction, decomposition, and reaction of gases in a reactor chamber.

CVD Apparatus Components

- Gas delivery system: Supplies reactants

- Reactor chamber: Where deposition occurs

- Substrate loading: Holds/removes substrates

- Energy source: Provides required heat/energy

- Vacuum & exhaust systems: Removes excess and by-products

- Process control equipment:  Monitors pressure, temperature, time

Steps in a CVD Process

- Transport of reactants by forced convection to the deposition region.

- Transport of reactants by diffusion from main gas stream through the boundary layer to wafer surface.

- Adsorption of reactants on wafer surface.

- Surface reactions including decomposition, migration, and incorporation

- Desorption of by products from the surface.

- Transport of byproducts by diffusion through boundary layer back to the main gas stream.

- Transport of byproducts by forced convection away from the deposition region.

Advantages of CVD

- High growth rates

- Suitable for materials which are hard to evaporate

- Good reproducibility

- Can grow epitaxial films

Disadvantages

- High temperatures required

- Complex setup

- Use of toxic/corrosive gases

Applications

- Protective coatings (wear, corrosion, erosion, temperature)

- Semiconductors and integrated circuits

- Sensors and optoelectronic devices

Plasma Enhanced Chemical Vapor Deposition (PECVD)

- Definition: PECVD is a type of chemical vapor deposition that uses plasma to enhance chemical reactions at lower temperatures.

- It is used to deposit thin films from gas state (vapor) to solid state.

- It converts reactant gases into a plasma, allowing thin films to be deposited on substrates from vapor.

Key Features

- The plasma is created by applying Radio Frequency or DC energy between electrodes.

- Plasma activates gases, leading to surface reactions.

- Films form on substrates as gases decompose and react.

Advantages Over Conventional CVD

- Lower deposition temperatures (Room temp to ~350°C vs 600–800°C in CVD)

- Better step coverage on uneven surfaces (trenches, walls, etc.)

- Tighter control of film properties

- High deposition rates

- Reduced thermal stress on devices

- Enables high performance bonding and electrical properties

How PECVD Works

- The substrate is placed between two electrodes in a deposition chamber

- Substrate is heated to 250–350°C

- Gases like silane (SiH₄) and ammonia (NH₃), with argon or nitrogen, are introduced

- Electrical discharge ignites plasma (100–300 eV), forming a glowing sheath

- Energized electrons collide with gas molecules in plasma

- Reactive species are transported via gas flow to substrate

- Chemical reactions form a film on the substrate

- By-products are pumped away

- Gas flow rates and temperatures affect film thickness, hardness, refractive index

Metal-Organic Chemical Vapor Deposition (MOCVD)

- Definition: It is a technique for depositing thin solid films (usually semiconductors) onto solid substrates (wafers) using organometallic compounds as precursors.

Applications

- Primarily used in Electronic and optoelectronic devices such as cell phones, LEDs and traffic lights, billboards, etc

Working Principle

- Group-III metalorganics and Group-V hydrides/alkyls act as the precursors.

- Carrier gases (H₂ or N₂) transport these precursor molecules onto heated substrate.

- Chemical reactions occur on a heated substrate (RF induction heating), forming a thin film.

- Process is also known as organometal vapor phase epitaxy.

- Reaction: Pyrolysis of metalorganics to form InN and GaN thin films

- Example Reactions:

- (CH₃)₃In + NH₃ → 3CH₄ (gas) + InN (solid)

- (C₂H₅)₃Ga + NH₃ → 3C₂H₆ (gas) + GaN (solid)

- Exhaust System

- Rotary and diffusion pumps

- Removes gaseous by products safely

- Material Requirements

- Sufficient volatility

- Stable at room temperature

- Good partial pressure

- Forms desired element + easily removable by-products